EXTRA PARTS

Need to 3d PRINT:

CASE FOR THE GYROSCOPE BREADBOARD AND ITS CAP

CASING FOR THE EMS/TENS MACHINE AND ITS RESPECTIVE CASE (For Servo breadboard housing)

- PARTS:

(Shaft x2)			Servo Gear x2

Bill of Materials (BOM)

Esp32 DevKitC(X2)- Arduino board for coding the project and bluetooth connectivity

MG90s Servo (X2)- Servo Motor for controlling Linear Actuators

MPU6050 Gyroscope (X1)- Gyroscope for determining head rotation

Half-size Breadboard (X2)-Standard Solderless Breadboards for connecting electronics

~15 Jumper Wires (M-M) – For wiring

AVCOO EMS/TENS Unit (x1) -For electric muscle stimulation

Standard EMS/TENS Unit Self-Adhesive Electrode Pads- (COMES WITH EMS DEVICE) Used to connect to EMS and apply electric pulses to body

AA 6V battery case (X1)- Used to power the SERVO breadboard

AAA 4.5V battery case WITH POWER SWITCH (X1)- Used to power the GYROSCOPE breadboard

Velcro strap--To wear the GYROSCOPE breadboard on head

Assembly- GYROSCOPE BOARD

X1 Breadboard

X1 ESP32

X1 MPU6050 Gyroscope

Jumper Wire

3d printed Gyroscope case and cap

Velcro Strap

Assembly- SERVO BOARD

X1 Breadboard

X1 ESP32

X2 MGS90 Servos

X1 AVCOO EMS/TENS Machine

Jumper Wires

Micro Servo Fastners

3d printed “PARTS” above

3d printed Servo case and cap

Setting up device: Master and Slave Bluetooth Coding

MASTER ESP32 (Gyroscope board)

Install VSCode and get the Platform IO Extension

Create a Project for the Master Code.

- Project settings: board- ESP32 DevkitC and code- Arduino

In the newly created project, install the MPU 6050 Library

- Also in the .IO file, add a monitor speed to 11520

Download code for Gyroscope board (ESP32 Master Code) and paste it in project.

Build and upload to the ESP32

SLAVE ESP32 (Servo board)

Create another Project for the Slave Code

- Same project settings

Install the ESP32 SERVO library along with the (FIND THE JSON BLUETOOTH LIBRARY THINGY)

- Again, add the Monitor to 11520 into the .IO file

Download code for the Slave board (ESP32 Slave Code) and paste it in project

Build and upload to the ESP32

*With both Master and Slave ESP32s done, they should be able to connect to each other, which will be signaled by a blue light appearing on both boards.